Component 2: Embedded System & Data Capture Pipeline

1. Narrative Description of Functional Concept Fragments

Each of the six core functions of the data capture pipeline has multiple candidate mechanisms. For each function, the best-fit mechanisms are described below with their operational narrative, illustrative logic, technical basis, and key operating conditions.

F1 — Camera Interfacing & Signal Acquisition

Function Definition: Establish a reliable communication link between the embedded Linux processor and the two USB UVC cameras (scene camera via /dev/video2; eye camera via its assigned /dev/videoX node), negotiate pixel formats and frame rates, and acquire raw image data into user-space memory.

Mechanism F1-A: V4L2 (Video4Linux2) Direct Device Access

Mechanism Overview: The application opens /dev/videoX device file nodes using the Linux V4L2 (Video4Linux2) kernel API. Through a sequence of ioctl system calls, the system negotiates the camera interface at the kernel level. Frames are retrieved using a memory-mapped streaming I/O (mmap) model: the application calls VIDIOC_QBUF to enqueue empty buffers and VIDIOC_DQBUF to dequeue filled frames. This provides direct, zero-copy access from the camera sensor to user-space.

Technical Basis and References:

- Linux kernel V4L2 API documentation: linux/videodev2.h — the industry-standard kernel interface for video capture on Linux.

- The Kexxu eye-tracking cameras have been confirmed as UVC-compliant via v4l2-ctl --list-devices enumeration, making V4L2 directly applicable without any driver development.

- "Pupil: An Open Source Platform for Pervasive Eye Tracking" (Kassner et al., Pupil Labs) — the reference system behind commercial eye-tracking hardware uses V4L2-based capture on Linux embedded platforms.

- "uvcvideo" kernel module: the standard Linux driver for USB Video Class devices, providing stable driver support for all UVC cameras including the Kexxu models.

Key Operating Conditions and Limits:

- Target frame rates: scene camera at 30 fps (1920×1080, MJPEG), eye camera at 60 fps (1280×800, MJPEG).

- Both cameras share a USB root hub; simultaneous peak bandwidth is approximately 60–80 MB/s, approaching USB 2.0's theoretical 60 MB/s sustained limit. Bandwidth contention is a validated risk (Unknown U1).

- Operating environment: Ubuntu Linux (Jetson default OS), indoor lab conditions (15–35°C ambient).

- Jitter constraint: per-frame dequeue jitter must remain below 2–3 ms to preserve timestamp quality.

#### Mechanism F1-B: GStreamer Multimedia Framework

#### Mechanism Overview: GStreamer constructs a directed acyclic graph (pipeline) of processing elements.

#### GStreamer internally handles format negotiation (caps negotiation), threading (separate OS thread per queue element), and buffer allocation. The framework integrates hardware acceleration plugins (e.g., nvv4l2h264enc for Jetson Orin Nano).

#### Technical Basis and References:

- GStreamer 1.x documentation — a widely-used open-source multimedia framework deployed in commercial embedded vision products.

- NVIDIA JetPack SDK includes GStreamer plugins specifically optimized for Jetson hardware (nvv4l2 plugin family), enabling hardware-accelerated encode/decode.

F2 — Frame Rate & Resolution Management

Function Definition: Control and sustain stable capture parameters (resolution, FPS, pixel format) across extended recording sessions under varying computational and thermal conditions.

Mechanism F2-A: Fixed Negotiation at Session Start (Static Configuration)

Mechanism Overview: At session initialization, the application explicitly sets all capture parameters via V4L2 ioctl calls before streaming begins:

- VIDIOC_S_FMT — sets pixel format (V4L2_PIX_FMT_MJPEG), width, and height.

- VIDIOC_S_PARM — sets the target frame rate (timeperframe numerator/denominator).

- V4L2 control API (VIDIOC_S_CTRL) — locks auto-exposure (V4L2_CID_EXPOSURE_AUTO = V4L2_EXPOSURE_MANUAL) and auto-white-balance to fixed values.

These settings are then held constant for the duration of the session, regardless of environmental changes.

Technical Basis and References:

- Linux V4L2 API — VIDIOC_S_FMT, VIDIOC_S_PARM, VIDIOC_S_CTRL are all part of the standardized V4L2 ioctl interface.

- Static configuration is the standard approach in scientific measurement instruments to eliminate parameter drift as a confound in data analysis.

- Validated: the scene camera has confirmed stable operation at 29.96 fps over 20.66 s in desk-based testing.

Key Operating Conditions and Limits:

- Resolution/FPS are locked at session start; any thermal throttling that reduces available CPU will cause frame drops rather than adaptive degradation.

- Effective for controlled lab sessions (15–35°C, ≤ 30-minute durations) as per Assumptions A9, A10, A12.

- Not suitable for varying-illumination wearable deployment (future phase); auto-exposure suppression may produce poor image quality in bright outdoor conditions.

Mechanism F2-C: Multi-Profile Capture with User Selection

Mechanism Overview: A configuration layer pre-defines a set of named capture profiles, each fully specifying all pipeline parameters:

| Profile | Eye Camera | Scene Camera | Expected Use |
| --- | --- | --- | --- |
| High Quality | 1280×800 @ 60 fps, MJPEG | 1920×1080 @ 30 fps, MJPEG | Lab controlled experiments |
| Balanced | 1280×720 @ 30 fps, MJPEG | 1280×720 @ 30 fps, MJPEG | Extended endurance sessions |
| Endurance | 640×480 @ 30 fps, MJPEG | 640×480 @ 30 fps, MJPEG | Thermal/battery constrained testing |

The user selects a profile via a command-line argument or configuration file before recording. The system loads the selected profile and applies all parameters identically to Mechanism F2-A.

Technical Basis and References:

- Multi-profile configuration enables systematic benchmarking of "Sampling Rate vs. Detection Accuracy" — a stated research contribution of this project (see Initial Notes).

- Aligns with the project deliverable of quantifying 30 Hz vs. 60 Hz eye detection performance differences.

Key Operating Conditions and Limits:

- Each profile must be empirically characterized for thermal behavior — the Balanced profile may be required if the High Quality profile causes sustained thermal throttling.

- No mid-session adaptation; profiles are fixed for the duration of a recording.

F3 — Timestamping & Temporal Alignment

Function Definition: Assign each acquired frame a precise, monotonically increasing timestamp and maintain temporal coherence between the dual camera streams to enable accurate gaze-to-world coordinate mapping.

Mechanism F3-A: Software Monotonic Clock Stamping

Mechanism Overview: Immediately upon each call to VIDIOC_DQBUF returning a filled frame, the capture thread records the current time using clock_gettime(CLOCK_MONOTONIC) in nanoseconds. The resulting timestamp is stored alongside the frame in a structured metadata log:

JSONL Entry Format:

{"frame_id": 1042, "camera_id": "eye", "timestamp_ns": 1234567890123456789,

"resolution": "1280x800", "nominal_fps": 60, "format": "MJPEG"}

Post-capture, inter-camera alignment is computed by matching eye-camera frames to the nearest-in-time scene-camera frame using the timestamp difference.

Technical Basis and References:

- CLOCK_MONOTONIC is immune to NTP jumps and wall-clock adjustments (POSIX specification); it is the correct clock for timestamping in real-time data acquisition.

- "PuRe: Robust Pupil Detection for Real-Time Pervasive Eye Tracking" — references software monotonic timestamps as the standard approach for frame-level timing in open-source eye trackers.

- This approach is already implemented and tested in the current development pipeline.

F4 — Buffering & Flow Control

Function Definition: Decouple frame acquisition from downstream processing and storage to absorb transient latency spikes from disk I/O, encoding delays, or compute load, preventing frame loss.

Mechanism F4-A: Per-Camera Ring Buffer (Bounded Queue)

Mechanism Overview: Each camera's dedicated capture thread writes acquired frames (as pointers to mmap buffer regions) into a fixed-capacity circular buffer. A separate consumer thread (encoder/writer) reads from the tail of the ring. When the ring is full, the oldest unprocessed frame is silently overwritten and a frame drop counter is incremented.

Technical Basis and References:

- Ring buffer (circular buffer) is the canonical data structure for producer-consumer decoupling in real-time systems (Cormen et al., Introduction to Algorithms, Chapter 10).

- Lock-free ring buffer implementation using std::atomic fence operations eliminates mutex contention between the capture and consumer threads.

- Used in virtually all professional video capture applications (FFmpeg, GStreamer queue elements, V4L2 multi-buffer streaming).

Key Operating Conditions and Limits:

- Ring buffer size must be empirically tuned: at 60 fps with MJPEG frames averaging ~50 KB each, a 16-frame ring buffer occupies ~800 KB — well within memory budget.

- Overflow is silent unless the frame drop counter is actively logged; monitoring is mandatory.

- Operating requirement: consumer thread must sustain average throughput ≥ 60 fps over any 5-second window; transient drops up to 100 ms are permissible.

Mechanism F4-C: V4L2 Kernel-Level Multi-Buffer (Zero-Copy DMA)

Mechanism Overview: The V4L2 VIDIOC_REQBUFS call allocates multiple kernel-managed DMA buffers (typically 4–8). The kernel camera driver cycles through these buffers automatically, filling each with a new frame via direct memory access (DMA) from the camera USB controller. The user-space application dequeues filled buffers, processes them, and re-enqueues them — creating a continuous zero-copy pipeline from camera hardware to user-space without intermediate memory copies.

Technical Basis:

- V4L2 mmap streaming I/O is the zero-copy capture model defined in the V4L2 specification; it is the fundamental mechanism underlying all V4L2 streaming applications.

- Zero-copy is essential at 60 fps: a 1280×800 MJPEG frame at ~50 KB × 60 frames/s = 3 MB/s DMA bandwidth; unnecessary memory copies would double this.

Key Operating Conditions and Limits:

- The number of kernel buffers is limited by driver and hardware (typically 4–32 for UVC devices). Buffer count is negotiated via VIDIOC_REQBUFS.

- If user-space processing consistently exceeds the inter-frame interval, kernel buffers will overflow and frames will be dropped at the driver level — invisibly from the application perspective unless VIDIOC_DQBUF is monitored for sequence number gaps.

- This mechanism is always active as long as V4L2 mmap streaming is used; it is the first layer of buffering regardless of additional user-space ring buffers.

F5 — Encoding & Local Storage

Function Definition: Compress and persist the captured dual-camera video streams and structured metadata to onboard storage in a format suitable for offline analysis.

Mechanism F5-A: MJPEG Pass-Through (Camera-Side Compression)

Mechanism Overview: The scene and eye cameras natively output MJPEG-compressed frames. These frames are written directly to disk in an AVI container (MJPEG codec ID 0x4745504A) without any re-encoding. Each frame is independently decodable (no inter-frame dependencies), and the camera hardware performs all compression internally, with zero CPU encoding overhead on the embedded processor.

Estimated Write Bandwidth

| Camera Type | Frame Size | Frame Rate | Bandwidth |
| --- | --- | --- | --- |
| Eye camera | 50 KB | 60 fps | 3.0 MB/s |
| Scene camera | 80 KB | 30 fps | 2.4 MB/s |
| Total |  |  | ~5.4 MB/s (Well within SSD capabilities) |

Technical Basis and References:

- MJPEG in AVI is a well-supported format in OpenCV (VideoWriter, FOURCC 'MJPG'), FFmpeg, and VLC.

- Assuming average MJPEG compression ratio of ~10:1, raw 1280×800 (≈ 1 MB) compresses to ~50–100 KB per frame — consistent with measured output from test recordings.

- "Evaluation of Video Compression Formats for Scientific Data Archival" — MJPEG's frame independence and moderate compression make it preferable to H.264 when individual frame access without decoding is required.

Key Operating Conditions and Limits:

- Storage requirement: ~5.4 MB/s × 1800 s (30-minute session) ≈ 9.7 GB per session. A 32 GB storage medium supports approximately 3 full sessions.

- MJPEG compression efficiency is 3–5× worse than H.264; for extended wearable deployment, this will become a constraint.

- No encoding latency — frames are written at capture rate; the storage I/O speed becomes the limiting factor.

Mechanism F5-B: Hardware H.264 Encoding (SoC Encoder)

Mechanism Overview: MJPEG-compressed frames from the camera are first software-decoded to raw YUV format, then passed to the SoC's dedicated hardware video encoding block (e.g., NVIDIA NVENC on the Jetson Orin Nano, or a Rockchip VPU). The hardware encoder produces a standard H.264 bitstream muxed into an MP4 container.

Technical Basis and References:

- NVIDIA JetPack SDK provides the nvv4l2h264enc GStreamer plugin and the Multimedia API for hardware-accelerated H.264 encoding on Jetson Orin Nano.

- H.264 achieves ~50:1 compression ratio (vs. MJPEG's ~10:1), reducing storage requirements by 5× for equivalent quality.

Key Operating Conditions and Limits:

- MJPEG decode adds ~5–10% CPU overhead per camera at 60 fps; hardware encoder adds minimal additional CPU.

- Encoding latency: 1–3 frames (~17–50 ms at 60 fps); acceptable for offline recording but adds complexity to real-time monitoring.

- Storage requirement: ~1.0–1.5 MB/s × 1800 s ≈ 1.8–2.7 GB per 30-minute session — a 5× improvement over MJPEG pass-through.

F6 — Data Export & Transfer

Function Definition: Move recorded video sessions and metadata from the embedded device to an external analysis workstation after recording completion.

Mechanism F6-A: Physical Wired Transfer (USB/Ethernet File Copy)

Mechanism Overview: After a recording session, the embedded device is physically connected to a workstation via USB 3.0 or Gigabit Ethernet. Files are copied using rsync (for incremental, checksum-verified transfer) or scp (SSH-encrypted copy):

# rsync example

rsync -avhP --checksum user@jetson:/mnt/recordings/session_2026_02_28/ \

/local/data/toyota_eyewear/

# Expected transfer time: 10 GB @ USB 3.0 (400 MB/s effective) ≈ 25 seconds

echnical Basis and References:

- rsync is the standard tool for reliable file transfer in embedded Linux research systems; checksum verification (-c flag) ensures data integrity.

- USB 3.0 sustained transfer rates of 300–400 MB/s are well within the bandwidth needed for session files of 2–10 GB.

- Aligned with the "offline-first development philosophy" stated in the project initial notes.

Key Operating Conditions and Limits:

- Requires a physical USB or Ethernet cable connection — appropriate for the current desk-based development phase.

- Transfer is manual (user-initiated); automated transfer scripts can be added as a shell script wrapper.

- Zero network dependencies; works in environments without WiFi infrastructure.

2. Preliminary Concepts Document

Each of the following architecture selects specific mechanisms for all six core functions and represents a coherent design philosophy:

| Concept ID | Architecture Name | One-Line Description |
| --- | --- | --- |
| C1 | V4L2-Native MJPEG Offline Pipeline | Minimal-complexity, zero-encoding-overhead pipeline using direct V4L2 access and MJPEG pass-through storage |
| C2 | GStreamer H.264 Hardware-Encoded Pipeline | Production-ready, storage-efficient pipeline using GStreamer framework with hardware H.264 encoding on Jetson Orin Nano |
| C3 | Hybrid V4L2 + UVC Metadata Research Pipeline | Research-optimized pipeline combining raw V4L2 timestamps with UVC hardware metadata for maximum temporal precision |

Architecture C1: V4L2-Native MJPEG Offline Pipeline

Concept ID: C1 Philosophy: Minimize complexity to achieve a reliable, debuggable baseline pipeline that validates the end-to-end architecture before adding encoding complexity.

Key Components

| Category | Component | Specification |
| --- | --- | --- |
| Sensing | Eye Camera | Kexxu USB UVC, 1280×800 @ 60 fps, MJPEG |
| Sensing | Scene Camera | ICT Camera USB UVC, 1920×1080 @ 30 fps, MJPEG |
| Actuation | IR LED Illuminator | Near-infrared illumination for pupil contrast |
| Computation | Embedded Board | Jetson Orin Nano (Ubuntu 22.04, 6-core ARM CPU, 8 GB RAM) |
| Communication | USB 3.0 Host | Dual-camera USB bus for capture; USB-A for wired export |
| Storage | Local SSD/eMMC | ≥ 32 GB, sustained write ≥ 20 MB/s |
| Software — Capture | V4L2 Direct API | Linux kernel V4L2 ioctl + mmap streaming |
| Software — Buffer | Ring Buffer | 16-frame circular buffer per camera (thread-safe) |
| Software — Storage | AVI Writer | OpenCV VideoWriter or custom MJPEG mux |
| Software — Metadata | JSONL Logger | Per-frame timestamp and metadata logging |
| Control Logic | Session Manager | Profile-based session initialization and shutdown |

Key Assumptions for C1

| Assumption ID | Assumption | Risk Level |
| --- | --- | --- |
| A1 | Dual USB UVC cameras are on separate USB controllers or sufficient bandwidth is available | Medium |
| A2 | Scene camera sustains 30 fps MJPEG at 1920×1080 (confirmed in testing: 29.96 fps) | Low |
| A3 | Eye camera sustains 60 fps MJPEG at 1280×800 continuously | Medium |
| A8 | Local storage provides ≥ 20 MB/s sustained write for dual-stream MJPEG (~5.4 MB/s total) | Low |
| A13 | Occasional frame drops (< 2%) are tolerable and will be logged | Low |
| A15 | CLOCK_MONOTONIC software timestamps are sufficient for initial pipeline validation | Medium |

Architecture C2: GStreamer H.264 Hardware-Encoded Pipeline

Concept ID: C2 Philosophy: Leverage the GStreamer framework and Jetson hardware encoder to produce storage-efficient H.264 sessions suitable for extended recording and production deployment.

Key Components

| Category | Component | Specification |
| --- | --- | --- |
| Sensing | Eye Camera | Kexxu USB UVC, 1280×800 @ 60 fps, MJPEG |
| Sensing | Scene Camera | ICT Camera USB UVC, 1920×1080 @ 30 fps, MJPEG |
| Actuation | IR LED Illuminator | Near-infrared illumination for pupil contrast |
| Computation | Embedded Board | Jetson Orin Nano (Ubuntu 22.04, 8 GB RAM) |
| Computation | Hardware Encoder | NVIDIA NVENC (nvv4l2h264enc) — hardware H.264 |
| Communication | USB 3.0 Host | Dual-camera USB bus; USB-A/Ethernet for export |
| Storage | Local SSD | ≥ 32 GB, segmented MP4 files (10-min rotation) |
| Framework | GStreamer 1.x | Pipeline construction, threading, format negotiation |
| Software — Encode | nvv4l2h264enc | NVIDIA hardware H.264 encoder plugin |
| Software — Mux | mp4mux | GStreamer MP4 container muxer |
| Software — Config | Profile YAML | Pre-defined capture profiles (High/Balanced/Endurance) |
| Control Logic | GStreamer State Machine | Pipeline state control (NULL→READY→PAUSED→PLAYING) |

Key Assumptions for C2

| Assumption ID | Assumption | Risk Level |
| --- | --- | --- |
| A7 | Jetson Orin Nano hardware encoder (NVENC) is available and compatible with nvv4l2h264enc plugin | High |
| A1 | Dual USB UVC cameras are on separate USB controllers or sufficient bandwidth is available | Medium |
| A3 | Eye camera sustains 60 fps MJPEG continuously | Medium |
| A10 | Recording sessions are 10–30 minutes (segmented files handle crash resilience) | Low |
| A17 | Consumers expect MP4 (H.264) output format — directly satisfied by this architecture | Low |

Architecture C3: Hybrid V4L2 + UVC Metadata Research Pipeline

Concept ID: C3 Philosophy: Research-optimized pipeline that combines software monotonic timestamps with hardware UVC metadata analysis to characterize timing precision and enable study-grade gaze-to-world mapping.

Key Components

| Category | Component | Specification |
| --- | --- | --- |
| Sensing | Eye Camera | Kexxu USB UVC, 1280×800 @ 60 fps, MJPEG |
| Sensing | Scene Camera | ICT Camera USB UVC, 1920×1080 @ 30 fps, MJPEG |
| Sensing | UVC Metadata Node | /dev/video3, V4L2_META_FMT_UVC, 10240-byte buffers |
| Actuation | IR LED Illuminator | Near-infrared illumination for pupil contrast |
| Actuation | BLE Module | Real-time gaze coordinate streaming to external device |
| Computation | Embedded Board | Jetson Orin Nano (Ubuntu 22.04, 8 GB RAM) |
| Communication | USB 3.0 Host | Dual-camera + metadata node USB bus |
| Communication | BLE 5.0 | Gaze data streaming (Jetson built-in or USB dongle) |
| Storage | Local SSD | ≥ 32 GB, dual AVI + extended JSONL metadata |
| Software — Timestamp Fusion | Drift Analyzer | Python/C++ tool correlating software and hardware timestamps |
| Software — Metadata Parser | UVC Header Parser | Extracts SCR/PTS from raw UVCH packet bitfield |
| Control Logic | Session Manager | Three-stream synchronization (eye video, scene video, metadata) |

Key Assumptions for C3

| Assumption ID | Assumption | Risk Level |
| --- | --- | --- |
| A5 | Scene camera /dev/video3 UVC metadata node is accessible and outputs valid UVCH data | High |
| A2 | PTS/SCR fields in the UVCH payload headers contain valid, firmware-populated timestamps | High |
| A1 | USB bandwidth supports three simultaneous V4L2 streams (eye video + scene video + metadata) | High |
| A15 | Hardware timestamps will show measurable improvement over software timestamps | Medium |
| A3 | Eye camera at 60 fps is stable for extended periods | Medium |

Initial Risk Identification Table

| Risk ID | Description | Source | Affected Concepts | Likelihood | Impact |
| --- | --- | --- | --- | --- | --- |
| R1 | USB Bandwidth Contention: Both cameras on the same USB root hub may saturate the bus, forcing FPS reduction | Assumption A6 (unvalidated) | C1, C2, C3 | Medium | High |
| R2 | UVC Metadata Invalidity: The /dev/video3 metadata PTS/SCR fields may be zero-filled (camera firmware does not populate them) | Assumption A5 violation | C3 | Medium | High |
| R3 | Eye Camera FPS Instability: The eye camera may not sustain 60 fps over 10–30 minutes; actual measured rate may be 45–55 fps | Unknown U7 | C1, C2, C3 | Medium | High |
| R4 | Thermal Throttling: Sustained dual-camera capture may cause the embedded board to thermally throttle, reducing CPU frequency and causing cascading frame drops | Unknown U4 | C1, C2, C3 | Medium | High |
| R5 | H.264 Hardware Encoder Unavailability: The current development platform may not have hardware H.264 encoding (NVENC is Jetson-specific) | Assumption A7 (unvalidated) | C2 only | High | Medium |
| R6 | Software Timestamp Jitter: CLOCK_MONOTONIC jitter may exceed 3 ms standard deviation under dual-stream CPU load, degrading inter-camera alignment | Unknown U8 | C1, C3 | Low | Medium |
| R7 | Storage I/O Bottleneck: The selected storage medium may not sustain the required ~5.4 MB/s dual-stream MJPEG write rate | Unknown U6 | C1, C3 | Low | High |
| R8 | Device Node Non-Determinism: Linux /dev/videoX numbering may change across reboots, requiring udev rules | Unknown U12 | C1, C2, C3 | Low | Low |
| R9 | Inter-Camera Clock Drift: Over 30-minute sessions, software timestamps from independent capture threads may drift >16.7 ms, making inter-camera alignment ambiguous | Unknown U3 | C1, C2 | Low | High |

Decision Criteria (Pre-Commit)

The following criteria will be used to evaluate and score candidate architectures. Criteria and weights are defined before scoring to prevent post-hoc rationalization.

| # | Criterion | Weight (%) | Derived From | Justification (Why It Matters) |
| --- | --- | --- | --- | --- |
| 1 | Capture Reliability (sustained dual-camera frame rate, drop rate < 2% over 30 min) | 30% | Core deliverable requirement: reliable synchronized video capture | Without sustained, drop-free capture, all downstream analysis (pupil detection, gaze mapping) is invalidated. This is the most fundamental requirement. |
| 2 | Timestamp Precision (per-frame timing accuracy and inter-camera alignment quality) | 25% | Research deliverable: gaze-to-world temporal mapping | Gaze accuracy depends directly on temporal alignment between eye and scene cameras. Poor timestamps make gaze vectors scientifically unreliable. |
| 3 | Implementation Risk & Complexity (number of unvalidated assumptions, implementation effort) | 20% | Project risk management; current development phase | Concepts requiring many unvalidated assumptions carry high schedule risk. The project is in early development; lower-risk concepts allow faster validated progress. |
| 4 | Storage Efficiency (GB per 30-minute session) | 15% | Long-term wearable deployment feasibility | Storage constraints become critical in field deployment. Higher efficiency reduces transfer time, cost, and hardware requirements. |
| 5 | Research Contribution Potential (enables novel measurements or publishable comparisons) | 10% | Project academic deliverables | The system must support research outputs such as the "Sampling Rate vs. Detection Accuracy" study and timestamp precision characterization. |

Score Rationale:

| Criterion | C1 Rationale | C2 Rationale | C3 Rationale |
| --- | --- | --- | --- |
| Capture Reliability | V4L2 direct + MJPEG pass-through = fewest processing layers, lowest failure modes | GStreamer adds pipeline complexity; hardware encoder path adds failure point | Three simultaneous USB streams increase bus contention risk; most complex |
| Timestamp Precision | CLOCK_MONOTONIC gives 0.5–3 ms accuracy; sufficient for 60 fps research | Same timestamp mechanism as C1; no improvement | UVC metadata could give sub-ms hardware timing, but only if PTS/SCR are populated (high risk) |
| Implementation Risk | All mechanisms validated or directly testable with current hardware; 0 hard blockers | Hardware encoder (A7) is unvalidated — if unavailable, C2 collapses to C1 without H.264 | Two critical unknowns (U2: UVC metadata validity) could invalidate the core differentiator |
| Storage Efficiency | MJPEG: ~9.7 GB / 30 min — storage fills fast in extended sessions | H.264: ~2.0 GB / 30 min — 5× improvement, enables longer sessions | MJPEG pass-through: same as C1 |
| Research Contribution | Enables FPS benchmarking via profile switching; established baseline | Enables H.264 vs. MJPEG compression quality study + FPS profiling | Enables novel UVC metadata timing study; highest academic novelty if metadata is valid |